# Quantifying the Contributions of Fe(VI), Fe(V), and Fe(IV) during Ferrate Oxidation of Micropollutants with a Kinetic Approach

Zhuang-Song Huang*, Xiao-Na Zhao, Yu-Lei Liu, Lu Wang, Jun Ma*

# (State Key Laboratory of Urban-rural Water Resource and Environment, School of Environment, Harbin Institute of Technology, Harbin, Heilongjiang, China)

Presenting Author: Zhuang-Song Huang

# Keywords: Water Treatment; High-Valent Iron Species; Micropollutants; Kinetics

# Introduction

Ferrate [Fe(VI)] oxidation of micropollutants involves Fe(V) and Fe(IV) intermediates, but their individual roles remain unclear, limiting mechanism-based optimization. Conventional kinetic models are hindered by uncertain pathways and secondary reactions. Using pyrophosphate to selectively suppress Fe(V) (Zhao et al., 2025), this study develops a kinetic framework to quantify the intrinsic second-order rate constants of Fe(VI) with organic contaminants (kFe(VI)-OCs) and to distinguish the contributions of Fe(VI), Fe(V), and Fe(IV), thereby clarifying the ferrate reaction selectivity.

# Materials and Methods

Potassium ferrate was prepared by wet chemical oxidation, and other chemicals were purchased. Batch kinetics experiments were conducted in 10 mM borate buffer with 1 mM pyrophosphate to suppress Fe(V). Kinetic fitting and simulation were carried out by Kintecus.

# Results and Discussion

By sequestering Fe(V) with pyrophosphate, three distinct trends were observed between the apparent second-order rate constant (kapp,OCs,PP) and the OCs/Fe(VI) molar ratio: positive, negative, or negligible dependence. Kinetic simulations showed that these trends are governed by the relative reactivity of Fe(IV) and enabled the establishment of quantitative relationships between kapp,OCs,PP and kFe(VI)-OCs. When kapp,OCs,PP increases with the OCs/Fe(VI) ratio, its lower-limit value approximates kFe(VI)-OCs. When a negative correlation is observed, the lower limit at high OCs/Fe(VI) ratios approaches 2kFe(VI)-OCs for two-electron contaminants, but remains close to kFe(VI)-OCs for one-electron contaminants. If kapp,OCs,PP is independent of the molar ratio, it remains near kFe(VI)-OCs (one-electron OCs) or 2kFe(VI)-OCs (two-electron OCs).

Based on these relationships, a kinetic approximation method was proposed to estimate kFe(VI)-OCs (i) examining kapp,OCs,PP variation with the OCs/Fe(VI) ratio under Fe(V) sequestration, (ii) identifying the dominant electron-transfer pathway via stoichiometric analysis, (iii) analyzing the quantitative relationship between kapp,OCs,PP and kFe(VI)-OCs; and (iv) determining the approximate value of kFe(VI)-OCs.

Figure 1. Relationship between kapp,OCs,PP and kFe(VI)-OCs.

Building on this, a targeted sequestered kinetic method was developed to quantify the individual contributions of Fe(VI), Fe(V), and Fe(IV): Fe(V) contributions were obtained using pyrophosphate, Fe(VI) contributions were calculated from the estimated kFe(VI)-OCs, and Fe(IV) contributions were derived by subtraction. Validation with methyl phenyl sulfoxide showed good agreement with kinetic model simulations.

Figure 2. Batch kinetics approach to identify the roles of different high-valent iron species.

# Conclusions

This study developed a targeted sequestered kinetic method to quantitatively distinguish the contributions of Fe(VI), Fe(V), and Fe(IV) during ferrate oxidation. Distinct relationships between apparent rate constants and OCs/Fe(VI) ratios reflect the relative reactivity of Fe(IV) and enable estimation of intrinsic Fe(VI) rate constants. The method overcomes limitations of conventional kinetic modeling and reveals selective oxidation by high-valent iron species, providing a basis for precision design and optimization of ferrate-based treatment processes.

# References

Zhao, X.-N., Huang, Z.-S., Chen, J., Liu, Y.-L., He, H.-Y., Cui, C., Ma, J. and Wang, L., 2025. Differential Impacts of Pyrophosphate on Ferrates(VI, V, and IV): Through Its Unique Inhibition to Identify Fe(V) Species. Environ. Sci. Technol. 59(15), 7768-7778.