# Differential Impacts of Pyrophosphate on the Oxidative Ability of Ferrates(VI, V, and IV)

Xiao-Na Zhao, Zhuang-Song Huang, Yu-Lei Liu, Lu Wang, Jun Ma*

# (State Key Laboratory of Urban-rural Water Resource and Environment, School of Environment, Harbin Institute of Technology, Harbin, Heilongjiang, China)

Presenting Author: Xiao-Na Zhao

# Keywords: High-valent iron species; Ligands; Chemical oxidation; Water treatment

# Introduction

Ferrate oxidation involves transient Fe(V) and Fe(IV) intermediates whose individual roles are difficult to resolve. Phosphate-based ligands can inhibit Fe(V) reactivity (Huang et al., 2018), but their effects on Fe(IV) remain unclear. Here, we systematically examine how pyrophosphate regulates the oxidation activity of Fe(VI), Fe(V), and Fe(IV), and interpret their reactivity changes using coordination geometry and electronic structure considerations.

# Materials and Methods

Potassium ferrate was prepared by wet chemical oxidation, and other chemicals were purchased. Experiments were conducted in 10 mM borate buffer. Reactions were initiated by adding ferrate stock solution. Kinetic fitting and simulation were carried out using Kintecus.

# Results and Discussion

Pyrophosphate, a strong non-reducing polydentate ligand, was selected to examine how coordination mode governs ferrate reactivity. We systematically investigated and summarized the influence of pyrophosphate on the oxidation activity of Fe(VI) and Fe(V). Subsequently, we established a sequential one-electron transfer process in the Fe(VI)-ABTS-Phenols system to targetedly produce Fe(IV) (Figure 1), enabling an in-depth investigation of interaction between Fe(IV) and pyrophosphate. The results reveal that pyrophosphate strongly inhibits Fe(V) oxidation, while it has minimal impact on the reactivity of Fe(VI) and Fe(IV).

Figure 1. The sequential one-electron transfer process in the Fe(VI)-ABTS-Phenols system.

Based on ligand field theory, pyrophosphate complexation can induce iron 3d orbital re-splitting, leading to spin electron rearrangement. Specifically, the hexa-coordinated Fe(V)-oxo complex ligated by pyrophosphate exhibits higher orbital energy, reducing its stability and effective collisions with contaminants. Whereas, the potential Jahn-Teller distortion of Fe(IV)-oxo complex could enhance its stability and preserve its significant reactivity (Figure 2). Given its selective inhibition on Fe(V) oxidation, pyrophosphate can emerge as a promising targeted quenching agent for Fe(V) species. Using methyl phenyl sulfoxide (PMSO) as the model compound, kinetic model simulation results can well fit the experimentally measured kinetic data of PMSO oxidation in the presence of pyrophosphate. This suggests that pyrophosphate can be used as the targeted quenching agent to explore the role of Fe(V) in ferrate system.

Figure 2. Possible configuration variation of iron-oxo complex after complexed by pyrophosphate.

# Conclusions

This study elucidates the distinct effects of pyrophosphate on high-valent iron species in ferrate oxidation. Pyrophosphate strongly suppresses Fe(V) reactivity but has little inhibition on Fe(IV) and Fe(IV), which can be explained by the differences in coordination geometry and electronic structure. These findings enable pyrophosphate to serve as a targeted quencher to identify Fe(V) contributions in situ, providing a new tool to distinguish Fe(V) and Fe(IV). The results also imply that phosphate-rich waters may reduce ferrate efficiency and highlight the need for strategies that favor Fe(IV) generation for optimized treatment.

# References

Huang, Z.-S., Wang, L., Liu, Y.-L., Jiang, J., Xue, M., Xu, C.-B., Zhen, Y.-F., Wang, Y.-C. and Ma, J., 2018. Impact of Phosphate on Ferrate Oxidation of Organic Compounds: An Underestimated Oxidant. Environ. Sci. Technol. 52(23), 13897-13907.