Untangling Natural and Anthropogenic Sources of DBP Precursors

Presenting Author: David Hanigan, University of Nevada, Reno

Keywords: Disinfection by-products; wildfire; NDMA; source apportionment

Disinfection byproduct (DBP) precursors arise from a wide range of natural and anthropogenic sources, yet their environmental relevance is often oversimplified. This presentation will demonstrate recent advances from my lab on precursor source materials and their importance.  A key point will be to emphasize the need for rigorous, evidence-based evaluation of precursor origins rather than assuming any single source is dominant. Prematurely ruling out specific sources can lead to misdirected research, inefficient treatment strategies, and misunderstanding of the chemical drivers of DBP formation.

NDMA serves as a clear example of this challenge. Wastewater has long been considered the primary source of NDMA precursors due to high laboratory yields from tertiary amines such as benzyldimethylamine (BDMA). Recent findings, however, show that many wastewater-associated precursors degrade rapidly in the environment and may contribute little to NDMA formation at actual drinking water intakes. Parallel studies demonstrate that benzalkonium chlorides (BACs), widely used antimicrobial compounds, are microbially transformed to BDMA in wastewater systems, contributing substantially to precursor loading yet these compounds also display limited environmental persistence.

In contrast, potable reuse systems reveal a different pattern: low-yield but persistent precursors such as dimethylamine (DMA) and chlorinated DMA (CDMA) can pass through RO-UV/AOP treatment trains and generate a large fraction of observed “reformation” or rebound NDMA. These findings highlight the importance of distinguishing between high-yield precursors that lack persistence and low-yield precursors that survive advanced treatment barriers.

Complementary research on wildfire impacts further demonstrates the variability and complexity of DBP precursor dynamics. Wildfires can sharply increase precursor loads and alter precursor reactivity in the short term, while long-term effects vary across landscapes and fire histories.

Collectively, these studies highlight a central principle: DBP precursor sources are diverse, dynamic, and context-specific. Effective DBP management requires avoiding assumptions about dominant sources and instead relying on systematic evaluation to identify which precursors truly drive formation under real-world conditions.

References:1-9

1.	Song, M., Wang, J., DeNicola, M., and Hanigan, D., Natural Vs. Anthropogenic Sources of N-Nitrosodimethylamine Precursors in Surface Water. Water Res, 2024. 265: p. 122313.

2.	Lacroix, A., Goli, T., Hanigan, D., and Pagilla, K., Formation, Speciation, and Temporal Variability of Dbps in Drinking Water Distribution Systems in the Context of Asr Operations and Extended Storage Periods. Chemosphere, 2024. 364: p. 143154.

3.	Lacroix, A., Goli, T., Hanigan, D., and Pagilla, K., Full-Scale Granular Activated Carbon and Chloramine Pretreatment of Drinking Water for Aquifer Storage and Recovery. ACS ES&T Water, 2024. 4(5): p. 2159-2169.

4.	Abusallout, I., Song, M., Chan, A., McKenna, E., Van Buren, J., McCoy, S., Ledvina, Z., Jeffrey, C., McCurry, D. L., and Hanigan, D., Bacterial Dealkylation of Benzalkonium Chlorides in Wastewater Produces Benzyldimethylamine, a Potent N-Nitrosodimethylamine Precursor. Water Research, 2024. 260.

5.	Stewart, K., An, D., and Hanigan, D., Reduction of Haloacetonitrile-Associated Risk by Adjustment of Distribution System Ph. Environmental Science-Water Research & Technology, 2023. 9: p. 2725-2732.

6.	Qian, Y., Shi, Y., Guo, J., Chen, Y., Hanigan, D., and An, D., Molecular Characterization of Disinfection Byproduct Precursors in Filter Backwash Water from 10 Drinking Water Treatment Plants. Science of the Total Environment, 2023. 856(Pt 1): p. 159027.

7.	Hickenbottom, K., Pagilla, K., and Hanigan, D., Wildfire Impact on Disinfection Byproduct Precursor Loading in Mountain Streams and Rivers. Water Research, 2023. 244: p. 120474.

8.	Song, M. R., Roback, S. L., Ishida, K. P., Wang, J. L., Plumlee, M. H., and Hanigan, D., Contribution of Dimethylamine to N-Nitrosodimethylamine Formation at Reverse Osmosis Water Reclamation Facilities. Environmental Science & Technology Letters, 2022. 10(1): p. 66-71.

9.	Song, M., Roback, S., Plumlee, M., and Hanigan, D. Dimethylamine Is an Important Ndma Precursor in Full Advanced Treated Water from Potable Reuse Facilities. American Water Works Association Water Quality Technology Conference. 2022. Tacoma, WA.