Differential Removal of Microcystis aeruginosa and Diatom Cells Using KMnO₄/NaClO Pre-oxidation Coupled with Powdered Activated Carbon–Coagulation

Ting Xie 1, Yalin Xu1, Wei Zhang1, Shaogang Liu 1, Zhimin Qiang 2

1 School of Materials and Environment, Guangxi Key Laboratory of Advanced Structural Materials and Carbon Neutralization, Guangxi Minzu University, Nanning 530105, P. R. China

2 School of Environmental Science and Engineering, Shanghai Jiao Tong University

Keywords：Cyanobacteria；Diatoms；Pre-oxidation；Powdered activated carbon (PAC)；Drinking water treatment

Cyanobacterial and diatom blooms pose distinct challenges to drinking water treatment. This study systematically compared the removal behaviors of Microcystis aeruginosa and the pennate diatom Nitzschia palea under potassium permanganate (KMnO₄) and sodium hypochlorite (NaClO) pre-oxidation, followed by powdered activated carbon adsorption and polyaluminum chloride coagulation–sedimentation (PAC–PACl), as well as their impacts on the formation of disinfection by-products (DBPs), algal toxins, and odor compounds.

The results showed that M. aeruginosa exhibited rapid growth, high biomass, and elevated polysaccharide and carboxyl contents, whereas N. palea displayed slower growth, lower biomass, and a rigid silica frustule, leading to distinct oxidative responses. KMnO₄ pre-oxidation (1.0 mg/L) was more effective for M. aeruginosa, achieving up to 96.8% chlorophyll-a reduction and significantly decreasing polysaccharide-, protein-, and humic-like functional groups (–OH, C=O, and C–O–C). In contrast, NaClO (1.0 mg/L) was more effective in inactivating diatom cells. However, excessive oxidant dosages caused severe cell lysis and the release of dissolved algal organic matter, thereby increasing the downstream treatment burden.

Coupling pre-oxidation with PAC adsorption and PACl coagulation–sedimentation substantially enhanced algal cell and organic matter removal, with optimal dosages of approximately 20 mg/L PAC and 10–20 mg/L PACl. Overall removal efficiencies exceeded 90% for M. aeruginosa and 85% for N. palea. The KMnO₄–PAC–PACl process showed superior cyanobacterial control performance, whereas NaClO–PAC–PACl was more effective against diatoms. Scanning electron microscopy (SEM) revealed progressive morphological deterioration from intact cells to shrinkage, rupture, and complete fragmentation. Antioxidant enzyme activities (SOD and CAT) initially increased and subsequently declined, indicating oxidative stress responses followed by irreversible structural collapse facilitated by adsorption and coagulation.

Pre-oxidation pathways strongly influenced DBP formation potential and odor risks during subsequent chlorination. NaClO pre-oxidation promoted high proportions of monochloroacetic acid and chloroform in M. aeruginosa waters and increased haloacetonitrile formation in diatom waters, accompanied by pronounced accumulation of odor compounds (e.g., β-cyclocitral, geosmin, and 2-MIB). In contrast, KMnO₄ pre-oxidation effectively reduced microcystin-LR from 6.3 to 0.3 μg/L while shifting DBP speciation toward bromochloroacetic acids and chloroform. The integrated oxidation–PAC–PACl process exhibited strong synergistic effects, reducing DBPs, algal toxins, and odor compounds by 70–95%. Three-dimensional fluorescence spectroscopy and FTIR analyses confirmed that KMnO₄/NaClO preferentially attacked easily oxidizable polysaccharides and proteins, disrupting extracellular polymeric matrices, while subsequent PAC adsorption and PACl coagulation effectively removed degraded algal organic matter and associated DBP precursors.

Overall, this study provides a mechanistic basis for oxidant-specific and algae-targeted process selection, offering optimized strategies to simultaneously control algal cells, DBP formation potential, and odor risks in algae-impacted drinking water sources.