Molecular Characteristics of Dissolved Organic Matter from Fast-Growing Eucalyptus Litter and Its Potential Impact on Disinfection By-Product Formation

Min Liu a，Yunpeng Zhang a，Kaihua Xie a，Qian Xu c，Kaisheng Diao a，Xuecai Tan a，Bernard A. Goodmana，Min Yang b，Shaogang Liu a, b, *，Huiyu Dong a, b, *

a Guangxi Minzu University, Nanning 530008, China

b Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing 100085, China

c Guangxi Lucheng Water Supply Group Co., Ltd., Nanning 530003, China

Email: liushaogang2005@163.com

Keywords: Dissolved organic matter (DOM); DOM reactivity; disinfection by-products; risk assessment

Abstract: Fast-growing Eucalyptus species have been widely cultivated in southern China due to their significant economic benefits. However, large-scale harvesting is frequently associated with the occurrence of “black water” phenomena in surrounding water bodies, primarily caused by the release of substantial amounts of dissolved organic matter (DOM) from Eucalyptus leaf litter into aquatic systems (Eucalyptus leaf-derived DOM, EL-DOM). DOM is a major precursor of disinfection by-products (DBPs) and readily reacts with disinfectants during drinking water chlorination, posing potential environmental and human health risks. In this study, the release behavior, molecular characteristics, and DBP formation potential of EL-DOM were systematically investigated, and its associated toxicity risks as well as the mitigation efficiency of water treatment processes were further evaluated. The results showed that elevated pH and temperature (10-30 ℃) significantly promoted the release of dissolved organic carbon (DOC) and dissolved organic nitrogen (DON) from Eucalyptus leaves. Excitation-emission matrix fluorescence spectroscopy identified four major fluorescent components, while high-resolution mass spectrometry (HRMS) revealed that EL-DOM was predominantly composed of lignin-like and carboxyl-rich alicyclic structures. Compared with Suwannee River natural organic matter (SRNOM), EL-DOM exhibited substantially higher DBP formation potentials under all three disinfectants tested, following the reactivity order of HOCl > NH2Cl > ClO2. Calculated toxicity assessments indicated that, in the absence of halide ions, cytotoxicity was mainly attributed to haloacetic acids. In contrast, in the presence of bromide or iodide, haloacetamides and haloacetic acids became the dominant contributors to toxicity, accompanied by a pronounced increase in brominated and iodinated DBPs, resulting in enhanced overall toxicity. Non-target HRMS screening combined with isotope pattern analysis tentatively identified more than 1,000 DBPs in total, among which 179, 439, and 287 aliphatic DBPs were detected following HOCl, NH2Cl, and ClO2 treatment, respectively. Furthermore, a simulated integrated water treatment train (coagulation-sand filtration-activated carbon adsorption-ultrafiltration) removed approximately 54% of DOC, with activated carbon adsorption and ultrafiltration contributing most significantly to the removal of DBP precursors. Overall, this study elucidates the release characteristics of DOM from fast-growing Eucalyptus litter, the mechanisms of DBP formation, and the effectiveness of water treatment processes, providing scientific guidance for source water protection and optimization of drinking water disinfection strategies in Eucalyptus-dominated regions.

References

1. Phan, C. K.; Wei, J. G.; Liu, F.; Chen, B. S.; Luo, J. T.; Yang, X. H.; Tan, X. F.; Yang, X. B. First Report of Corynespora Leaf Spot of Eucalyptus in China. Plant Dis. 2015, 99(3): 419–419.

2. Liu, B.; Qu, Z.; Ma, Y.; Xu, J.; Chen, P.; Sun, H. Eucalyptus Plantation Age and Species Govern Soil Fungal Community Structure and Function Under a Tropical Monsoon Climate in China. Front. Fungal Biol. 2021, 2: 703467.

3. Yang, G.; Wen, M.; Deng, Y.; Su, X.; Jiang, D.; Wang, G.; Chen, Y.; Chen, G.; Yu, S. Occurrence Patterns of Black Water and Its Impact on Fish in Cutover Areas of Eucalyptus Plantations. Sci. Total Environ. 2019, 693: 133393.

4. Franklin, H. M.; Doederer, K.; Neale, P. A.; Hayton, J. B.; Fisher, P.; Maxwell, P.; Carroll, A. R.; Burford, M. A.; Leusch, F. D. L. Terrestrial Dissolved Organic Matter Source Affects Disinfection By-Product Formation during Water Treatment and Subsequent Toxicity. Environ. Pollut. 2021, 283: 117232.