The majority of disinfection byproducts (DBPs) remains unknown and may pose toxic risks. By mimicking the molecular interactions of toxicants with biomolecules, we developed a solid-phase microextraction (SPME) reactivity-directed analysis (RDA) method for identifying unknown toxic DBPs. The probe 4-mercaptophenylboronic acid (MPA) was covalently immobilized onto the SPME fiber coating through cis-diol bonding. After capturing toxic DBPs through thiol reactions, the pH-responsive cleavage of boronate ester allowed for releasing the MPA-DBP adducts. The boron isotope pattern (1:4) and characteristic fragments derived from MPA moiety allowed for efficient prioritization of the adducts. Compared to the current homogeneous approach using liquid extraction, this heterogeneous method exhibited the advantages of biomimetic toxicity focus, effective pretreatment, and enhanced detection performance. The application in chlorinated simulated source water discovered thirty DBP-MPA adducts, in which the molecular structures of seven adducts, i.e., seven DBPs, were identified, including a known DBP chloroacetic acid and six newly found DBPs. Two of them were confirmed as 1-penten-3-one and methacrolein using authentic standards. The toxicity and health risk of newly found DBPs were predicted using computational tools, indicating their toxicity contributions in disinfected waters. This study represented an important advance in the identification methodology for unknown toxic DBPs.