# Mechanistic modeling and optimization of solar chlor-photo-Fenton for wastewater reclamation with disinfection by-product mitigation

P. Soriano-Molina1, *, L. Nofuentes-Gómez1, D. Rodríguez-García1, S. Belachqer-El Attar1, J. L. García Sánchez1, J. L. Casas López1, J. A. Sánchez-Pérez1

# (1 Solar Energy Research Centre (CIESOL), Joint Centre University of Almeria-CIEMAT, Almería, Spain)

Presenting Author: P. Soriano-Molina

# Keywords: pathogens; micropollutant; wastewater treatment; kinetic model; safe water reuse.

# Introduction

Wastewater reclamation and reuse are effective solutions for sustainable resource management, and increasingly restrictive regulations have made efficient quaternary treatments in wastewater treatment plants (WWTPs) imperative (EU Regulation 2020/741; EU Directive 2024/3019). In this context, the solar chlor-photo-Fenton (SCPF) process has emerged as a promising approach for the simultaneous disinfection and removal of micropollutants from WWTP secondary effluents, while mitigating the formation of disinfection by-products. Investigating the underlying reaction mechanisms is essential for developing reactor design tools, as well as for process control and optimization. Accordingly, this work focuses on the development of a mechanistic model of the SCPF process to optimize the operation of raceway pond reactors (RPRs) and ensure efficient wastewater reclamation.

# Materials and Methods

Experiments were conducted using WWTP secondary effluents spiked with benzotriazole (BZT, 50 μg/L), as a model micropollutant. The effects of temperature (15, 25, and 35 °C), ferric nitrilotriacetate (Fe³⁺-NTA, 0.1 and 0.2 mM) and sodium hypochlorite concentrations (NaOCl, 0.134, 0.268, and 0.402 mM), and UVA irradiance (10, 20, 30, and 40 W/m2) on BZT removal were evaluated, together with the inactivation of naturally occurring Escherichia coli (E. coli, 5.4·102 – 3.3·104 CFU/100 mL) and Clostridium perfringens (C. perfringens, 1.8·102 – 7.2 103 CFU/100 mL) in 10-cm deep stirred-tank reactors placed in a solar simulator. The kinetic parameters were estimated from experimental data assuming perfect mixing and batch mode operation. Then, a model-based optimization was conducted incorporating polynomial equations into an optimization routine to determine the optimal reagent concentrations and reaction times for different environmental scenarios and treatment targets.

# Results and Discussion

Based on process phenomenology, a semi-empirical mechanistic model was proposed, achieving a good balance between the description of photochemical and microbiological processes. The model involves different reaction pathways: chlorination, thermal Fe3+-NTA decomposition, Fenton-like and solar photo-Fenton, coupling bacterial inactivation with micropollutant removal. It successfully fitted the experimental data, with normalized root mean square errors <20% (Figure 1). The optimal reagent concentrations to minimize reagent costs and maximize treatment capacity varied with the treatment target, with values close to 0.05 mM of Fe³⁺-NTA, 0.30 mM of H₂O₂, and 0.67 mM of NaOCl (Figure 2), whereas the optimal reaction times ranged from 50 to 90 min.

Figure 1 (A) BZT removal, and E. coli and C. perfringens inactivation by SCPF at 10-cm liquid depth, 25 °C and 40 W/m2 UVA irradiance, with 0.402 mM NaOCl, 1.47 mM H2O2 and 0.1 mM Fe3+-NTA. Lines represent model estimations. (B) Optimal reagent concentrations, unitary operating cost, and reaction time (RT) for the treatment targets based on C. perfringens concentration: 5-log10 reduction related to the WWTP influent (Target A), and a residual concentration ≤1 CFU/100 mL in the treated water (Target B), both with >80% of BZT removal.

# Conclusions

The model contributes to the mechanistic understanding of the SCPF process and enables optimization, thereby supporting the control and automation of RPR operation, ensuring compliance with new regulations while promoting economic and environmental sustainability.

# Acknowledgements

The authors acknowledge the contribution of the NEREIDAS (P_LANZ_2024/011, PPIT-UAL), CAFIRA (PID2023-152519OB-I00) and INACUA (PLSQ_00330) projects.

# References

EU Directive 2024/3019 of the European Parliament and of the Council of 27 November 2024 concerning urban wastewater treatment. Off. J. Eur. Union, 12.12.2024.

EU Regulation 2020/741 of the European Parliament and of the Council of 15 May 2020 on minimum requirements for water reuse. Off. J. Eur. Union (177/32), 5.6.2020.