Formation of Trichloronitromethane from Algal Organic Matter Triggered by Far-UVC Irradiation: Mechanisms and Implications for Water Treatment

Xiaodi Duan1,∗, Yue Sun1, Liang Tang1

(1 Key Laboratory of Organic Compound Pollution Control Engineering (MOE), School of Environmental and Chemical Engineering, Shanghai University, Shanghai, China)

Presenting Author: Xiaodi Duan

Keywords: Far-UVC; Algal Organic Matter; Trichloronitromethane; Singlet Oxygen; Nitrate

Introduction

Far-ultraviolet light at 222 nm (UV222) from KrCl excilamps is gaining attention for its high efficiency in microbial inactivation and micropollutant degradation, coupled with enhanced human safety. However, its application in water impacted by harmful algal blooms, which release significant amounts of algal organic matter (AOM), requires careful evaluation. AOM is a potent precursor for nitrogenous disinfection by-products (N-DBPs), particularly the highly cytotoxic and genotoxic trichloronitromethane (TCNM). While UV254 is widely used, the distinct photochemistry of UV222, including its higher photon energy and potential for generating reactive species in water, may uniquely influence DBP precursor formation. This study aimed to systematically evaluate the TCNM formation potential (TCNM-FP) from extracellular (EOM) and intracellular (IOM) AOM fractions after UV222 exposure and examine TCNM-FP in real surface and drinking waters.

Materials and Methods

A collimated beam apparatus equipped with a 222 nm KrCl excimer lamp or 255 nm UV-LEDs (as a proxy for conventional UV254) was used for irradiation. Microcystis aeruginosa was cultivated, and EOM and IOM were extracted. Reactive species (singlet oxygen, hydroxyl radical, superoxide) were identified and semi-quantified using electron paramagnetic resonance (EPR) spectroscopy. For TCNM-FP tests, AOM solutions (DOC = 5 mg C/L) and real water samples were irradiated at various UV fluences (0–600 mJ/cm²), followed by chlorination with NaOCl (24 h, 25°C, dark). TCNM was quantified using GC-TOFMS. The fluorescent composition of organic matter was characterized by three-dimensional excitation-emission matrix (3D-EEM) spectroscopy with fluorescence regional integration (FRI). The effects of model precursors (humic acid, fulvic acid, tryptophan, and alanine) and added nitrate on TCNM-FP were also investigated.

Results and Discussion

EPR and probe experiments confirmed the stable generation of 1O2 during UV222 irradiation of ultrapure water, a process dependent on dissolved oxygen and water molecules. HO• and O2•− were also detected. This reactive environment significantly altered AOM. UV222 irradiation markedly promoted TCNM-FP from EOM, which increased to a maximum (~186.5 µg/L at 400 mJ/cm2) before declining at higher fluences, likely due to precursor degradation. In contrast, UV255 irradiation produced no TCNM from any AOM fraction, and no TCNM was detected from IOM under UV222.

3D-EEM and FRI analysis revealed that the increase in TCNM-FP from EOM correlated positively with the proportion of humic acid-like components. Model compound tests confirmed that humic acid, but not fulvic acid, tryptophan, or alanine, generated TCNM after UV222 irradiation and chlorination, identifying humic acid-like substances as key precursors.

While IOM alone did not yield TCNM, the addition of a relatively low concentration of nitrate (1 mg-N/L) to IOM solutions enabled TCNM formation after UV222 irradiation, enhancing precursor generation likely through photolysis to reactive nitrogen species. Higher nitrate concentrations reduced TCNM-FP, attributed to light screening effects.

In real waters, UV222 irradiation also increased TCNM-FP in surface water (SW) and drinking water source water (DW-RAW), with trends similar to EOM. However, no TCNM was formed in treated drinking water after filtration (DW-AF), where conventional processes had effectively removed DBP precursors, particularly humic acid-like substances.

Conclusions

This study demonstrates that UV222 irradiation, through the generation of 1O2 and other reactive species, can significantly promote the formation of TCNM precursors from algal organic matter, a risk not observed with conventional UV254. The risk is primarily associated with humic acid-like components in extracellular AOM. The presence of nitrate at environmental levels can further enhance this risk for intracellular AOM. Crucially, conventional drinking water treatment processes, such as filtration, can mitigate this risk by removing the relevant precursors. These findings highlight the need for a balanced evaluation of UV222 technology, considering its promising disinfection efficacy alongside potential DBP formation risks, especially in algal-impacted water sources.

References

Fang, J., Ma, J., Yang, X., Shang, C., 2010. Formation of carbonaceous and nitrogenous disinfection by-products from the chlorination of Microcystis aeruginosa. Water Res., 44(6), 1934-1940.

Payne, E.M., Langelan, E.G., Linden, K.G., 2025. High absorbance of nitrate leads to surprising effects on hydroxyl radical formation during 222 nm UV treatment. Water. Res., 123754.

Zhang, X., Zhang, X., Li, H., Ao, X., Sun, W., Li, Z., 2024. Reactive oxygen species generated in situ during carbamazepine photodegradation at 222 nm far-UVC: unexpected role of H2_22​O molecules. Environ. Sci. Technol., 58(42), 19070-19079.