Exploring the correlations between physicochemical parameters and the presence of opportunistic pathogens in drinking water using a machine learning approach

Hyeok Kim1, †, Juliana Marks2, Hyun-Suk Oh1,3, Rabin Bhattarai4, and Helen Nguyen*2

1Department of Environmental Engineering, Seoul National University of Science and Technology, 232, Gongneung-ro, Nowon-gu, Seoul, 01811, Republic of Korea

2Department of Civil and Environmental Engineering, The Grainger College of Engineering, University of Illinois at Urbana-Champaign

3 Institute of Environmental Technology, Seoul

National University of Science & Technology, Seoul 01811, Republic of Korea

4Department of Agricultural and Biological Engineering, The Grainger College of Engineering & College of Agricultural, Consumer and Environmental Sciences, University of Illinois at Urbana-Champaign, Urbana, IL 61801, USA

*Corresponding and presenting author: Helen Nguyen E-mail:  thn@illinois.edu

Keywords: opportunistic pathogens; machine learning; drinking water distribution systems.

Introduction

Opportunistic pathogens (OPs) in drinking water distribution systems (DWDS) and premise plumbing pose health risks. Understanding how physicochemical parameters influence OP occurrence is critical for effective control strategies. In this study, machine learning (ML) was applied to explore relationships between physicochemical parameters and OPs, including Legionella spp., Pseudomonas spp., Mycobacterium spp., and Vermamoeba vermiformis.

Materials and Methods

Two datasets from previous studies were used to develop ML models. The first dataset, collected by (Zhang et al., 2024), included 111 samples from DWDS and premise plumbing. The second dataset consisted of 211 samples collected from a home plumbing system simulator; however, samples with point-of-use filters installed were excluded, and only 76 samples collected from the entry point, water tank, and recirculation were used in this study (Gomez-Alvarez et al., 2023). Four types of opportunistic pathogens—Legionella spp., Mycobacterium spp., Pseudomonas spp., and Vermamoeba vermiformis—were measured using quantitative polymerase chain reaction (qPCR) and the copy numbers of the targeted genes were taken as the concentrations of these OPs.

Three models, eXtreme Gradient Boosting (XGBoost), Random Forest (RF), and Support Vector Regression (SVR), were optimized using cross-validation, and Shapley Additive exPlanations (SHAP) were employed to interpret variable influences. Before applying machine learning, the data were normalized to scale the variables to a common range and to mitigate the influence of outliers (Cabello-Solorzano et al., 2023). The entire dataset was divided into training (80%) and testing (20%) prior to normalization. The training data were normalized using the Min–Max normalization method, and the test data were transformed based on the minimum and maximum values obtained from the training data to prevent data leakage (Bouke & Abdullah, 2023).

Results and Discussions

Fig 1. Combined-datasets RF model developed for Legionella spp., with SHAP analysis explaining the relationships between each parameter and Legionella spp. (a) Performance of RF models. (b) SHAP summary plot based on the RF model showing feature contributions.

Three parameters, pH, free chlorine and water temperature all influence inactivation of Legionella spp. However, pair-wise correlations between Legionella spp. concentration and each of these parameters yielded a low Spearman coefficients of 0.41, -0.31, and 0.29, respectively. Given this poor  pair-wise correlation, we explored the complex relationship between Legionella spp. and six water quality parameters by applying three ML algorithms (XGBoost, RF, and SVR). Of the 157 data points in combined dataset, 125 were used for model training, and the remaining 32 for testing. Among the three algorithms, the RF model showed the best performance in test (R² = 0.93 train, 0.80 test) (Fig. 1a). XGBoost (R² = 0.95 train, 0.75 test) and SVR (R² = 0.77 train, 0.69 test) models showed slightly lower performance than the RF model. This train–test split allowed an independent evaluation of model generalizability to unseen data. Despite the relatively limited parameters in the dataset (pH, free chlorine, water temperature, and concentrations of three Ops), the models demonstrated stable predictive performance on the test data.

SHAP analysis was performed using the combined dataset RF model, which achieved the best performance in testing to determine the contribution of each parameter to the ML model predictions. This method allows interpretation of the complex links between physicochemical parameters and Legionella spp. concentration. The results showed that pH (range: 6.11-9.47) was the most influential predictor, followed by free chlorine and water temperature (Fig. 1b). Furthermore, the RF and XGBoost models showed similar SHAP feature contribution patterns, indicating that the identified key predictors are robust across these algorithms (Fig. 1b).

We subsequently examined the SHAP plots to identify the causes of water quality parameters on the predicted concentrations of Legionella spp. First, the high pH values are predominantly associated with positive SHAP values (red dots on the right in Fig. 1b) or higher pH leads to higher predicted concentration of Legionella spp. At higher pH values (range: 8.31-9.15; mean: 8.69 ± 0.29), free chlorine shifts toward hypochlorite ions (pKa=7.5) with lower disinfection efficacy against Legionella spp. (Kuchta et al., 1983). This shift results in reduced inactivation and consequently higher detected concentrations, as reflected by the positive SHAP values. On the other hand, negative SHAP values were associated with low pH (range: 6.57-7.82, mean: 6.97 ± 0.38) (blue dots on the left in Fig. 1b), suggesting that low pH is associated with lower predicted concentration of Legionella spp. Second, the concentrations of free chlorine also correlate with Legionella spp. concentrations. Fig. 1b shows mostly positive SHAP values associated with low free chlorine concentration (range: 0-1.45; mean: 0.30 ± 0.40 mg/L), suggesting that free chlorine concentration around 0.3 mg/L leads to higher predicted concentration of Legionella spp. This range of free chlorine concentration is likely insufficient for effective inactivation of Legionella spp.  In contrast, a few negative SHAP values associated with high free chlorine concentration (range: 0.5-2.39; mean: 1.53 ± 0.73), which are sufficient to inactivate Legionella spp., resulting in a lower predicted concentration. We found a previous study (Cervero-Aragó et al., 2015), reporting that for planktonic Legionella pneumophila, 4-log inactivation by 0.2 or 0.5 mg/L free chlorine can take between 2-20 min, depending on the strains. Another study (Shen et al., 2017) showed that inactivation was reduced when L. pneumophila cells were associated with drinking water biofilms. Given these previous findings, the correlations found by ML are reasonable.  Third, water temperature influences the inactivation of Legionella spp. The negative SHAP values were associated with higher temperature (range: 21.3-35.2°C; mean: 28.19 ± 3.13°C), suggesting that higher temperature leads to lower predicted concentrations of Legionella spp., likely due to faster inactivation by free chlorine within this temperature range.  A similar observation was   reported previously(Kuchta et al., 1983), showing that L. pneumophila was inactivated more quickly as the temperature increased from 4 to 32°C. On the other hand, positive SHAP values were associated with lower water temperature (range: 21.1-41.3°C; mean: 25.90 ± 5.15°C), at which slower reactions with free chlorine may occur, resulting in higher predicted concentrations of Legionella spp. We also used SHAP dependence plots to show that the highest concentrations of Legionella spp., are found when pH is above 8.0, (i.e., above pKa of chlorine) and free chlorine concentration is below 0.5 mg/L. Water temperature generally decreased with increasing temperature up to approximately 30°C due to faster inactivation of Legionella spp, followed by a tendency to increase at higher temperatures due to faster decay of free chlorine

In addition to three common parameters for drinking water quality, pH, free chlorine and temperature, the influence of three microbes: Mycobacteria spp, V. vermiformis, and Pseudomonas spp, on Legionella spp. concentrations were also explored. As shown in Fig. 1b, the SHAP values for V. vermiformis and Legionella spp. concentrations indicate that higher concentrations of V. vermiformis are associated with positive SHAP values, whereas lower concentrations are associated with negative SHAP values. This finding suggests high concentrations of V. vermiformis lead to higher predicted concentrations of Legionella spp. On the contrary, low concentrations of V. vermiformis are associated with lower predicted concentrations for Legionella spp. The cause of these correlations is the ability of Legionella spp.  to reside within V. vermiformis and gain protection from environmental stresses. Notably, Legionella spp. that multiply within V. vermiformis have been reported to be more resistant to free chlorine than those replicating within Acanthamoeba castellanii (Chang et al., 2009; Shen et al., 2017). We also found similar correlations between the concentrations of Mycobacteria spp. and Legionella spp., suggesting that these two bacteria are likely to be found together, probably due to ability of both bacterial species to escape disinfectants by hiding inside V. vermiformis. In contrast, the presence of Pseudomonas spp. does not contribute to the prediction of Legionella spp., with the SHAP values clustered around zero.

Conclusions

We demonstrated that ML algorithms can be used to characterize the relationships between OPs and the complex environmental conditions present in DWDS and premise plumbing. Free chlorine, water temperature, and pH were identified as key parameters associated with Legionella spp. Specifically, the worst protection is when pH is above 8.0 and free chlorine concentration is below 0.5 mg/L regardless of water temperature. Increasing water temperature until 30°C correlates with lower predicted concentration of Legionella spp. but higher than 30°C may lead to faster decay of free chlorine and therefore less protection.  The strong mutual influence observed in ML models between V. vermiformis and Legionella spp or Mycobacterium spp. reflected the host–parasite relationship.

References

Bouke, M. A., & Abdullah, A. (2023). An empirical study of pattern leakage impact during data preprocessing on machine learning-based intrusion detection models reliability. Expert Systems with Applications, 230(6), 120715. https://doi.org/10.1016/j.eswa.2023.120715

Cabello-Solorzano, K., Ortigosa de Araujo, I., Peña, M., Correia, L., & J. Tallón-Ballesteros, A. (2023). The Impact of Data Normalization on the Accuracy of Machine Learning Algorithms: A Comparative Analysis. Lecture Notes in Networks and Systems, 750 LNNS, 344–353. https://doi.org/10.1007/978-3-031-42536-3_33

Cervero-Aragó, S., Rodríguez-Martínez, S., Puertas-Bennasar, A., & Araujo, R. M. (2015). Effect of common drinking water disinfectants, chlorine and heat, on free Legionella and amoebae-associated Legionella. PLoS ONE, 10(8). https://doi.org/10.1371/journal.pone.0134726

Gomez-Alvarez, V., Ryu, H., Tang, M., McNeely, M., Muhlen, C., Urbanic, M., Williams, D., Lytle, D., & Boczek, L. (2023). Assessing residential activity in a home plumbing system simulator: monitoring the occurrence and relationship of major opportunistic pathogens and phagocytic amoebas. Frontiers in Microbiology, 14. https://doi.org/10.3389/fmicb.2023.1260460

Kuchta, J. M., States, S. J., Mcnamara, A. M., Wadowsky, R. M., & Yee2, R. B. (1983). Susceptibility of Legionella pneumophila to Chlorine in Tap Water. In APPLIED AND ENVIRONMENTAL MICROBIOLOGY (Vol. 46, Number 5). https://journals.asm.org/journal/aem

Shen, Y., Huang, C. H., Lin, J., Wu, W. J., Ashbolt, N. J., Liu, W. T., & Nguyen, T. H. (2017). Effect of Disinfectant Exposure on Legionella pneumophila Associated with Simulated Drinking Water Biofilms: Release, Inactivation, and Infectivity. Environmental Science & Technology, 51(4), 2087–2095. https://doi.org/10.1021/acs.est.6b04754

Zhang, C., Sienkiewicz, N., Struewing, I., Mistry, J. H., Buse, H., Hu, Z., & Lu, J. (2024). Reconsider the burn: The transient effect of a chlorine burn on controlling opportunistic pathogens in a full-scale chloraminated engineered water system. Science of the Total Environment, 933. https://doi.org/10.1016/j.scitotenv.2024.172690