Unraveling the role of far UVC and UV/DCC pretreatment in mitigating volatile DBPs from DOM during water disinfection: Insights from non-targeted high-resolution mass spectrometry and interpretable machine learning

Qiyuan Suna, *

a. College of Environmental and Resource Sciences, Fujian Normal University, Fuzhou, Fujian 350117, China

Abstract

Ultraviolet (UV) based pretreatment and disinfection process would change the molecular components of dissolved organic matter (DOM) in raw water, thereby altering the generation potential of disinfection byproducts (DBPs) in drinking water treatment. This study investigated the effect of four representative wavelengths of UV irradiation (including far UVC) and UV/DCC process on DOM components transformation and DBPs generation in real surface water through Fourier transform-ion cyclotron resonance mass spectroscopy (FT-ICR MS) combined with interpretable machine learning. The results demonstrated that all the four types of UV irradiation promoted DOM transforming from high molecular weight (MW) and aromatic components to low MW and saturated one, which easily reacted with bromide ions and free chlorine to generate bromine-chlorine mixed DBPs. Notably, UV222 and UV254 tended to decompose nitrogen- and sulfur-containing components of DOM and reduce their reaction pathways, resulting in a lower yield of nitrogen-containing DBPs (N-DBPs). By contrast, UV310 and UV365 preferred to degrade DOM components with high H/C, resulting in a high dose of carbon-containing DBPs (C-DBPs). Besides, DOM exhibits higher aromaticity and oxidation states directly correlating with increased volatile DBPs formation potential and cytotoxicity in UV/DCC process. Aromatic/tannin components in DOM drive carbonaceous and nitrogenous DBPs (C-DBPs and N-DBPs) formation. CHOCl molecules of DOM with low H/C and high unsaturated and oxygenated structures, act as key THM precursors.

KEYWORDS: Dissolved organic matter; Volatile disinfection byproducts; Far UVC; UV/DCC; FT-ICR MS; Machine learning